INTRODUCTION

Over the last decade, Adsorptive Stripping Voltammetry (AdSV) has evolved into a very versatile and powerful electroanalytical technique. This method is aimed to extend the scope of the sensitivity and selectivity of stripping analysis toward numerous analytes (organic molecules and metal chelates exhibiting surface-active properties [1]. Besed on the wide range of applications reported during the recent years, the adsorptive concept appears to be the most universal stripping approach. When the given substance contains an electrochemically reducible or oxidisable group, the peak current on the voltammetric curve is recorded after completion of the accumulation period.

Electroanalytical procedures have been proposed for the determination of trace metals because they can be a valid option in the multicomponent analysis of several species; they can also be used for natural water, especially seawater [2].   The sensitivity and selectivity of these methods can be improved either by the introduction of new types of electrodes or by the development of new analysis procedures.   Electrochemical  methods include stripping voltammetric techniques, principally anodic stripping (ASV) and adsorptive cathodic stripping (AdSV) voltammetry, which  show  numerous  advantages,  such  as minimum  sample pretreatment,  high  speed  of  analysis,  good  performance  with saline matrix, low cost, and good selectivity and sensitivity [1,3].

The development of AdSV methods has allowed the determination of many trace metals; this was not limited to amalgam-forming elements such as ASV.  The method is based on the interaction between metal ion and a ligand, followed by the  accumulation  of  the  complex  onto  the  electrode  surface, from which it is subsequently stripped during the cathodic voltammetric scan.  The ligand forms a complex with the target metal,  which  is adsorbed  and later  reduced  on the electrode. The adsorption of the complex is often related to the solubility [4,5]. In addition, the size of the metal complex determines the surface coverage, and thus the sensitivity. Therefore, it is necessary to study the applicability of new ligands that may increase the sensitivity and selectivity of trace metal ion determination.

A possible limitation of AdSV is that sometimes only one or two metals can be determined in a single scan, depending on the metal  specificity  of  the  ligand  used  for  the  determinations [4]. Because of the great sensitivity enhancement obtained with AdSV methods, several complexing agents have been studied for use during the adsorptive collection of complexes, but there are only few studies about simultaneous metal determination [5-8].

These proposed methods use different ligands: for example, dimethyl glyoxime [8] to detect Co, Zn, Cd and Ni, but with high detection limits (2.3 –  5.1 µg/L) or substituted catechols [5]  that lack  much  sensitivity  (>5 µg/L). Moreover,  a  mixture  of ligands has been proposed for the multi-elemental determinations of several elements (Cu, Pb, Cd, Ni, Co and Zn) in seawater [4]. The detection limits of 0.2 µg/L for Cu(II), 0.4 µg/L for Pb and 0.12 µg/ L  for Cd were obtained using dimethyl glyoxime and 8-hydroxyquinoline as ligand by AdSV.  Therefore, there is a great interest in developing new ligands with adequate complexing properties, which may perform the simultaneous determination of several metals at trace levels.

Trace heavy metals are very important in the environment due to their serious toxicity although present at very low concentrations. The presence of heavy metals in the environment, including food ingredients is very dangerous, because of high level of toxicity [9,10]. Adsorptive  stripping  voltammetry  was  chosen  as  an  alternative  method  of  analysis because it has many advantages such as: high salt content of sea water does not interfere,  high sensitivity, low detection limit in ug/L,  simple and easy sample preparation, rapid analysis, less infra structure [11,12].  In addition, with this method it is possible to learn chemical species of heavy metals [13,14] which can not be done with other methods.

This method can be used  for the simultaneous determination of Cd and Pb in food samples by using 2-mercaptobenzothiazole as complexing agent [15]. Simultaneous determination of Cd, Cu and Pb  in sea water by AdSV[16]   the simultaneous determination of copper, bismuth and lead in the presence of   thymolphthalexone [17} and simultaneous determination of trace of cadmium and zinc by using 5-phenyl-1,2,4-triazol-3-tion (PTT) as complexes agent[18]. Simultaneous determination  of ultra trace levels of copper and cadmium in food and water samples with luminal as a chelating agent [19], simultaneous determination of lead and cadmium in the presence of morin as a complexing  agent [20] and simultaneous determination of copper, lead, cadmium and zinc by adsorptive stripping voltammetry   in  the presence of murexide [21].   Almost all methods of determination of metals in very small amounts require considerable time on pre-concentration step prior to measurement. In adsorptive stripping voltammetry pre-concentration time is  short, generally less than one minute [22].

The    present    paper    describes    an    adsorptive    stripping   voltammetry    procedure    for determination cadmium, copper, lead and zinc in water samples   using Bis (N,N-bis(carboxymetyl)aminometyl) fluorecein or Calcein  as   a   complexing agent. Calcein is important  chelating  agents  because  of their donor  properties  in  coordinating  metals  and  their  ability  to behave as polydentate ligands in neutral and deprotonated forms. They are widely employed for the   fluorometric determination of metals,[23] being good analytical reagents due to their metallochromic  properties [24]. Because of their interesting properties, they can be applied in the determination of heavy metals by adsorptive stripping voltammetry [3].

This study was used calcein as a chelating agent for simultaneous of  analysis Cd, Cu, Pb and Zn metals by adsorptive stripping voltammetry. The structure of calcein can be seen in Figure 1.

Figure 1. Structure of Calcein (Fluorexon)

To overcome the problem mentioned above a better condition of adsorptive stripping voltammetry is needed to get a selective and sensitive method. The  parameters studied were: variations of calcein concentration, variation of pH solution, accumulation potential and accumulation time. To determine the precision, accuracy, sensivity and linearitas  of  method, relative standard deviation (RSD), recovery, limit of detection and correlation coefficient were determined.  The  method  at  optimum  condition  was  applied    for  the  direct  determination of Cd(II), Cu(II), Pb(II) and Zn(II) in seawater  around of Bungus and water tap.

EXPERIMENTAL

The materials used for this study were: NH4Cl, calcein, concentrated HNO3, concentrated HCl, acetate buffer, NH4OH, nitrogen gas, doubly distilled water, Whatman filter paper and samples (sea water, water tap and water of Lubuk Minturun). Various chemicals were purchased from Merck and the aqueous solution of Cd, Cu, Pb and Zn 1000 mg/L was prepared by using the analytical grade Merck product.

The instrument used in this study were: 797 Computrace with Metrohm HMDE working electrode, a reference electrode Ag/AgCl/KCl 3 M and the Pt electrode as a counter electrode, pH meter models 80 Griffin (Griffin & George Loughborough, UK), analytical balance Mettler AE 200, Toledo OH-USA and glassware commonly used in laboratory.

Methods: The aim of this study was to obtain the optimum condition of each metal ion Cd, Cu, Pb and Zn by adsorptive stripping voltammetry. Therefore studied the effect of the following parameters namely, variations of concentration calcein of 0.2 mmol/L calcon to 0.9 mmol/L, variation of pH 3 to pH 9, accumulation potential of -0.2 V to -1.1 V and accumulation time variation of 20 s to 100 s. To determine the precision and accuracy of method relative standard deviation (RSD) and recovery were determined. Procedure used in this study according were reported previously [11].

Sample preparation: Sea water samples were obtained from a beach around Bungus, Padang City, West Sumatera, Indonesia. The samples were placed in plastic bottles previously washed with 1 % HNO3 solution. In the laboratory they were filtered through a 0.45 µm membrane filter and acidified with HNO3 to pH close to 2. Tap water samples were obtained from the laboratory, Department Chemistry Andalas University, river water were obtained from Lubuk Minturun Padang City. Before the analysis all the samples were digested by UV irradiation for 90 min at 90 °C (10.0 mL of sample with 100 µL of 30 % H2O2).

RESULT AND DISCUSION

Preliminary experiments were carried out to identify the general features which characterise the behaviour of the metal ions-calcein systems on a hanging mercury drop electrode (HMDE). Fig. 2 shows adsorptive stripping differential pulse voltammograms of Cd(II), Cu(II), Pb(II) and Zn(II)-calcein system. Curve (a) shows the voltammograms of mixed solution containing 10 µg/L of cadmium, copper, lead and zinc in the absence of calcein under optimum conditions. Very small cathodic peaks were found at - 0.13, - 0.38, -0.56 and -0.98 V for copper, lead, cadmium and zinc, respectively. Curve (b) shows the adsorptive stripping voltammmograms of mixture of metal ions (cadmium, copper, lead and zinc) and calcein system. Four separated peaks for the reduction of cadmium, copper, lead and zinc complexes with calcein were observed that was due to the difference in the thermodynamic stability of their complexes. Comparison of the voltammograms revealed that height of the cadmium, copper, lead and zinc peaks depends not only on the duration of the preconcentration step but, in the presence and absence calcein, also the stripping initial potential,which revealed the adsorptive nature of the response [9].

Fig. 2.  Adsorptive stripping voltammetry for Cd(II). Cu(II), Pb(II) and Zn(II)-calcein system.(a) Differential

pulse adsorptive stripping voltammograms in mixed solution of Cd(II), Cu(II), Pb(II) and Zn(II) 10

µg/L in the absence of calcein. (b) the adsorptive stripping voltammmograms of mixture of metal ions

(cadmium, copper,  lead and zinc) in the presence calcein

.

In this work, a new sensitive and rapid adsorptive voltammetric method for simultaneous determination of ultra trace levels of cadmium, copper, lead and zinc was developed based on adsorptive stripping voltammetry of cadmium, copper, lead and zinc complexes with calcein

Effect variation of concentration calcein

The effect of calcein concentration on the sensitivity of the proposed method was also studied cathodic stripping peak current of Cd- Cu, Pb and Zn-calcein complexes increased with increasing calcein concentration up to 0.6 mM, at concentration higher than 0.6 mM where both peaks diminish as a consequence of a full electrode surface coverage. An optimum calcein concentration of 0.6 mM was selected for further experiment (Fig.3)

Figure 3. Effect variation of concentration calcein on peak current (Ip)

Condition : 10 mL mixed solution of Cd, Cu, Pb dan  Zn 10 µg/L; 0.2 mL calcein ; 0.2 mL KCl

0.1 M; accumulation time 60 s; accumulation potential -0.7 and pH 4.

Effect variation of pH

The  inffluence of pH on the cathodic stripping peak currents of cadmium, copper, lead and zinc was studied in the pH range of 3.0 – 9.0. It was found that at pH 4.0 the peak currents of cadmium, copper, lead and zinc were at their maximum value. Thus pH 4.0 was adopted for further studied (Fig. 4). The decrease in the response for cadmium, copper, lead and zinc, is due to competitive formation of metal hydroxide.

Figure 4. Effect variation of pH on peak current (Ip)

Condition : 10 mL mixed solution of Cd, Cu, Pb dan  Zn 10 µg/L; 0.2 mL calcein 0.6 mM ;

0.2 mL KCl   0.1 M; accumulation time 60 s and accumulation potential -0.7 V

Effect variation of accumulation potential

The effect of accumulation potential on  the stripping peak current of  the complex was   examined over   the  potential  range of  - 0.1 V to  - 0.9V. The  voltammograms of the above system in  the accumulation potential range indicate that the best peaks separation   took place when  the  accumulation potential  of     - 0.7 V was   used for  the pre-concentration step. As  shown in  Fig. 5,  the peak current increased with changing potential from -0.1 V to -0.7 V, probably due to the increased accumulation of the complex  on the electrode surface The peak current decreased at potentials more negative than   -0.7 V, so  this potential was  used in all experiments.

Figure 5. Effect variation of accumulation potential on peak current (Ip)

Condition : 10 mL mixed solution of Cd, Cu, Pb dan  Zn 10 µg/L; 0.2 mL calcein 0.6 mM ;

0.2 mL KCl   0.1 M; accumulation time 60 s and pH 4

Effect variation of accumulation time

Fig. 6 shows plots of the cathodic peak current in adsorptive stripping voltammetry versus accumulation time for cadmium, copper, lead and zinc. The time requared for accumulation of the metal complex, with less time requared for higher concentration. At mentioned before, calcein concentrations that are higher than requared may cause inhibition of the chelate adsorption by competitive coverage by the free ligand. An increase in the stripping peaks current with accumulation time was observed for Cd -, Cu, Pb and Zn-calcein complexes. As is expected for adsorption processes, the dependence of the peak current on the accumulation time is limited by saturation of electrode, resulting in the current reaching a plateau at high accumulation times, as shown in Fig. 6

Thus,  deposition time of 60 s was  used throughout,  as  it  combines good  sensitivity and relatively short analysis time.

Figure 6. Effect variation of accumulation time on peak current (Ip)

Condition : 10 mL mixed solution of Cd, Cu, Pb dan  Zn 10 µg/L; 0.2 mL calcein 0.6 mM ;

0.2 mL KCl   0.1 M; accumulation potential – 0.7 V  and pH 4

Analytical  ﬁgure  of merit

Linear range and limit of detection: Under the optimized conditions, a linear relationship between the reduction peak current of the iron, cobalt, nickel and chromium complexes and the concentration of Cd(II), Cu(II), Pb(II) and Zn(II) was obtained. The result  can be seen in Table 1. To verify the linear relationship between peak current and metal concentrations, four calibration curves were plotted under optimum conditions. The calibration equation, obtained by least-squares method, is Ip = 3.936C +161.2 (r2 = 0.986) for Cd(II), Ip = 4.327C + 303.6 (r2 = 0.988) for Cu(II), Ip = 5.012C + 146 (r2 = 0.988) for Pb(II) and Ip = 2.521C + 113.5 (r2 = 0.985) for Zn(II) determination, where C is the concentration of metal ions (µg/L) and Ip is the peak current (nA). The stripping peak current of Cd(II)-calcein, Cu(II)-calcein, Pb(II)-calcein and Zn(II)-calcein complexes were found to be directly proportional to be the cadmium, copper, lead and zinc concentration in the range of range 0.2 -1 35.0; 0.2 - 110.0;  0.2 - 100.0 and 0.2 - 110.0 µg/L, respectively. The limit of detection is calculated of three times standard deviation of blank solution devided with slope  (3SD/B) under optimum conditions were obtained:  1.021, 0.987, 0.972 and 0.957 µg/L of Cd(II), Cu(II), Pb(II) and Zn(II),  respectively.

Table  1. Analytical parameters  obtained for simultaneous analysis for Cd(II), Cu(II),

Pb(II) and Zn(II) ions.

| Determination |  | pH | Slope/nA µg/L | Intercept/nA | R2 | Linier Range/µg/L | Detection limit/µg/L |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Simultaneous determination | Cd(II) | 4 | 1.433 | 217.0 | 0.882 | 10 - 110 | 1.387 |
|  | Cu(II) |  | 1.408 | 203.5 | 0.937 | 10 - 100 | 0.766 |
|  | Pb(II) |  | 1.342 | 128.5 | 0.904 | 10 - 110 | 1.437 |
|  | Zn(II) |  | 1.433 | 217.0 | 0.882 | 10 - 100 | 0.983 |

Relative standard deviation (RSD) :

Relative standard deviation is used to look at the precision of the method is the same operating conditions. The determination of the relative standard deviation performed at optimum conditions have been defined above. At the optimum conditions with eight times replicates (n = 8) measurements of 10 µg/L standard solution of Cd(II), Cu(II), Pb(II) and Zn(II) were: 0.39, 0,74, 1.35 and 0.41%, respectively. According to the AOAC method, the value of relative standard deviation was smaller than 8 % at concentration of 10µg/L. That mean this method has a high degree of precision because the RSD obtained for each metals was smaller than 8 %.

Recovery:

Recovery is used to determine the level of accuracy of this method, it was necessary to determine the value of recovery. Samples of known concentration were added the amount of standard solution and then compared with the concentrations of samples and standards added. In this study the recovery of sea water samples studied was taken from Bungus Padang City. The results of recovery of Cd(II), Cu(II), Pb(II) and Zn(II) by adsorptive stripping voltammetry at the optimum conditions, its value obtained were: 99.11, 99.88, 98.76 and 99.79 %, respectively. According to the AOAC method, the percent recovery for the solution with the concentration of 50 µg/L was in the range of 70-110 %. The result can be seen in Table  2. That is, this method has high accuracy [25].

Table 2. Simultaneous determination of Cd, Cu, Pb and Zn in sea water Bungus, Padang

City

| Added(µg/L) |  |  |  | Found (µg/L) |  |  |  | Recovery (%) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cu | Pb | Cd | Zn | Cu | Pb | Cd | Zn | Cu | Pb | Cd | Zn |
| 0 | 0 | 0 | 0 | 18,7±2.7 | 12,2±1,3 | 38,1±1.7 | 184,3±1.7 | - | - | - | - |
| 10 | 20 | 30 | 40 | 27,8±1.4 | 31,8±1,6 | 69,3±1.2 | 225,4±2.3 | 97 | 99 | 102 | 101 |
| 20 | 30 | 40 | 10 | 37,9±3,5 | 41,5±1.8 | 78,9±2.1 | 193,7±2.2 | 98 | 98 | 101 | 99 |
| 30 | 40 | 10 | 20 | 48,6±2.3 | 51,2±2.8 | 48,9±2.3 | 203,1±1.3 | 99 | 98 | 102 | 99 |

Interference studies

Possible interference by other metals with the  adsorptive  stripping voltammetry of  cadmium, copper, lead and zinc was  investigated  by   the  addition  of   the  interfering ion   to   the  solution containing 10.0 mg/L  of these metals using the optimised conditions. The results of this study are  summarised in Table  3. Based  on the results, it was  found that most of the foreign ions  did not  interfere  for cadmium, copper, lead  and zinc determination.

Table 3. Tolerance ratio of interfering ions in the determination

of 10 µg/L of Cd, Cu, Pb and  Zn

| Ions | Tolerance limit (mg/L) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Cu | Pb | Cd | Zn |
| Na+, Al+, Ca+, Li+, K+, Ba2+, Cr+3, Co2+, Ni2+, Cl-, F-, Br-, SO42-, I- | 10 | 10 | 10 | 10 |
| Cu2+ | - | 10 | 1 | 1 |
| Pb2+ | 10 | - | 10 | 10 |
| Cd2+ | 10 | 10 | - | 10 |
| Zn2+ | 1 | 1 | 10 | - |
| Fe3+ | 1 | 1 | 1 | 1 |

Application

The  proposed method was  successfully applied for  determination of  cadmium, copper, lead and zinc in  several water   samples. The Cd(II), Cu(II), Pb(II) and Zn(II) contents of  water samples, such as  tap  water, river water, lab  water and sea  water  were determined using the recom- mended procedure under optimum conditions, using the standard addition method. The  results presented in  Fig. 8, show the high sensitivity of the proposed method

Fig. 8.  Voltammogram  of Cd, Cu, Pb and Zn in sea water samples  from Bungus Padang.

City

CONCLUSION

It can be concluded that the optimum conditions for the simultaneous measurement of Cd(II), Cu(II), Pb(II) and Zn(II) by adsorptive stripping voltammetry were : concentration of calcein 0.6 mM, pH 4.0,  accumulation potential of -0,7 V (vs. Ag/AgCl) and accumalation time of 60 s. Under the optimum conditions the measurement peak current is proportional to the concentration of Cd(II), Cu(II), Pb(II) and Zn(II) in the range 0.2 - 110 µg/L ; 0.2 – 100 µg/L ; 0.2 – 110 µg/L ; and 0.2 – 100 µg/L respevtively. The limit of detection of the method was 1.3871 µg/L for Cd(II), 0.7663 µg/L for Cu(II), 1.4372 µg/L for Pb(II) and 0.9832 µg/L for Zn(II). The relative standard deviation were 0.84%, 0.25%, 0.43% and 0.54% for Cd(II), Cu(II), Pb(II) and Zn(II) for ten replicates (n = 10) measurements of mixture solution 10 µg/L of Cd(II), Cu(II), Pb(II) and Zn(II). The method was successfully applied for simultaneous determination of Cd(II), Cu(II), Pb(II) and Zn(II) in tap, sea water and lake water samples of Maninjau Agam, West Sumatra.

ACKNOWLEDMENTS

One of the authors (Deswati) would like to thanks to Directorate General of Higher Education, Ministry of Research and Technology, Higher Education who has funded this research in accordance with the Letter Agreement Implementation Fundamental Research Assignment Number : 003/UN.16/PL/D-FD/2014.

References

1.  C. Locatelli and G. Torsi, Microchem. J, 65, 203 (2000).

2.  J.  Wang,  “Stripping  Analysis-Principles,  Instrumentation and Application”,

VCH, Deerfield Beach, (2006).

3.  J.  A.  Jurado-González,   M.  D.  Galindo-Riaño,   and  M.García-Vargas, Anal.

Chim.  Acta, 487, 229 (2003).

4.  C. Colombo and C. M. G. van der Berg, Anal. Chim. Acta,  337, 29 (1997)

5.  J. Limson and T. Nyokong, Anal. Chim. Acta, 344, 87 (1997).

6.   A. A. Ensafi, T. Khayamian, A. Benvidi, and E. Mirmomtaz, Anal. Chim. Acta

561, 225 (2006).

7.  E. Shams, A. Babaei, and M. Soltaninezhad,  Anal. Chim. Acta, 501, 119 (2004).

8.  A. A. Ensafi and K. Zarei, Talanta, 52, 435 (2000).

9.   S. Abbasi, K. Khodarahmiyan and F. Abbasi, Food Chem, 128, 254 (2011).

10.  H. A. Ahmed, M. Jamal, Am. J. App. Sci, 4, 594 (2009).

11.  Deswati, H. Suyani and Safni, Indo. J. Chem, 12, 20 (2012).

12.  S. Zang and W. Huang, Anal. Sci, 17, 983 (2001).

13.  M.S. El-Shahawi, A.S. Bashammakh, A.A. Al-Shibaai, S.O. Bahaffi    and E.H. Al-

Gohani,  Electroanal, 23, 747 (2011).

14.  R. Jugade and A. P. Joshi, Anal. Sci, 22, 571 (2006).

15.   S. Abbasi, K. Khodarahmiyan and F. Abbasi,  Food Chem, 128, 254 (2011).

16.  Deswati, H. Suyani, Safni, U. Loekman and H. Pardi,  Indo. J. Chem, 13, 236 (2013).

17.  A. Babaei, I. Shams and A. Samadzadeh, Anal. Sci, 22, 955 (2006).

18.  F. M. Shemirani, A. Rajabi, Asghari and H. Milani, Can. J. Anal. Sci and Spect 50,

175 (2005).

19.  A. Shahryar, A. Bahirae and F. Abbasai, Food Chem, 129, 1274 (2011)

20.  N. Edgar, V. Arancibia, R. Rios and C. Rojas, Int. J. Electrochem. Sci, 7, 5521(2012).

21. Deswati, C. Buhatika. H. Suyani, Emriadi, U. Loekman and H. Pardi, Int. J. Res. Chem. Environ,  4, 143 (2014).

22.  M. K. Amini  and M. Kabiri, J, Iran of Chem. Soc, 2, 32 (2005).

23.  J. Liu, B. Zhang, B. Wu, Y. Liu, and X. Yu, Rare Met, 25, 184 (2006).

24.  A. S. A. Zidan, Phosphorus,  Sulfur Silicon Relat. Elem, 178, 567(2003).